# iSIMM looks beneath the Faroes basalt using low-frequency, long-offset seismic from Ocean Bottom Seismometers and Q-Marine streamers.

# The iSIMM working group†. Presented by Dr Roman Spitzer

Industry and university goals coincide in the integrated Seismic Imaging and Modelling of Margins (iSIMM) project whereby low frequency seismic energy, together with high-density, long-offset, single-sensor recordings, are used to penetrate volcanic overburdens and illuminate both the deep structure of magmatic margins and sub-basalt sediments in adjoining basins.

The North Atlantic continental margin was formed by continental break-up in the early Tertiary above the Iceland mantle plume. This led to stretching and thinning of the continental lithosphere, accompanied by massive magmatism, producing flood basalts which covered pre-existing sediments and extensive igneous intrusion. The seismic components of iSIMM seek to develop a structural model from seabed to Moho to image the stretched crust and the extruded, intruded and under-plated igneous material, and to use these constraints in magmatic margin basin modeling.

In summer 2002, seismic data were acquired successfully over two magmatic ocean margins: the Hatton-Rockall margin and the margin to the northeast of the Faroe Islands, using a combination of towed streamers and Ocean-Bottom Seismometers (OBS). We focus on the surveys over the Faroe margin, where a 385 km profile was acquired using OBS and by WesternGeco using three Q-Marine streamers. The profile passes over the complete range of basalt environments from no basalt at all on the southeast flank of the Faroe-Shetland Trough, through increasing basalt cover and the basalt escarpment, over the Fugløy Ridge, across the continent-ocean transition east of the Faroe Islands and out over oceanic crust. Each acquisition used a source array tuned for low frequencies, building upon reported experiences of imaging below shallow basalts west of Ireland, and seismic characterisation of Lower Series basalts in the Lopra borehole on the Faroes.

In this paper, we point out the key features of the acquisition and the processing, we review the results to date from both OBS and streamer data and we show how the two datasets complement each other in estimating the base of the basalt sequence and characterising deeper intervals. A comparison with an earlier long-offset survey shows the benefit of the low frequency bandwidth and we summarise the impact on the stratigraphic interpretation of features below top basalt.

Publications

White, R.S., Spitzer, R., Christie, P.A.F., Lunnon, Z., Maresh, J. & iSIMM Working Group. Seismic imaging through basalt flows on the Faroes shelf. Faroese Society of Sciences and Humanities (Proceedings of Faroes Exploration Conference, Tórshavn, 4-5 May 2004), submitted.

Hoare, R., Schearer, P., Langridge, A., Saragoussi, E., Christie, P. & iSIMM Team. Imaging sub-basalt with deep towed streamer: a case study from the Faroe Islands. Faroese Society of Sciences and Humanities (Proceedings of Faroes Exploration Conference, Tórshavn, 4-5 May 2004), submitted.

Smith, L.K., White, R.S., Kusznir, N. J. & iSIMM Team. Structure of the Hatton-Rockall Basin and adjacent continental margin. Proc. 6th Petroleum Geology Conference: N.W. Europe & Global Perspectives, in press.

Roberts, A.W., White, R.S., Lunnon, Z.C., Christie, P.A.F., Spitzer, R. & iSIMM Team, (2004). Rifting and magmatism on the Faroes margin, North-West Europe and Global Perspectives: Proceedings of the 6th Petroleum Geology Conference, Geological Society of London, in press.

Spitzer, R., White, R.S., and Christie, P.A.F., (2004). Sub-basalt imaging along the iSIMM profile – integration of surface and ocean bottom data. 66th Mtg.: Eur. Assn. Geosci. Eng., Paris, Exp Abs. D042.

Christie, P., Langridge, A., White, R., Lunnon, Z., Roberts, A.W. and the iSIMM team, (2004). iSIMM looks beneath basalt for both industry and university research. Ext. Abs. presented at 5th Petroleum Geophysics Conference, Hyderabad, India

Spitzer, R., White, R.S., and Christie, P.A.F., (2003). Enhancing sub-basalt imaging using parabolic tau-p transformation. The Leading Edge, 22(12), 1184-1201.

Lunnon, Z., Christie, P., and White, R., (2003). An evaluation of peak and bubble tuning in sub-basalt seismology: modelling and results from OBS data. First Break, 21(12), 51-56.

White, R.S., Christie, P.A.F., Kusznir, N.J., Roberts, A., Davies, A., Hurst, N., Lunnon, Z., Parkin, C.J., Roberts, A.W., Smith, L.K., Spitzer, R., Surendra, A., and Tymms, V., (2002). iSIMM pushes frontiers of marine seismic acquisition. First Break, 20(12), 782-786.

† The iSIMM working group comprises Robert S White, Nick Kusznir, Philip Christie, Alan Roberts, David Healey, Roman Spitzer, Vincent Tong, Alex Chappell, Jennifer Eccles, Rosie Fletcher, Neil Hurst Zoë Lunnon, Craig Parkin, Alan W. Roberts, Lindsey Smith and Vijay Tymms.